R2-H2>en

R2-H2#sh run

Building configuration...

Current configuration : 1241 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R2-H2

!

!

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX152465DD-

!

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback0

ip address 2.2.2.2 255.255.255.0

!

interface GigabitEthernet0/0

description LINK TO ISP

ip address 192.168.20.1 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/1

description LINK TO HQ CORE

ip address 192.168.10.2 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

log-adjacency-changes

redistribute bgp 65010 subnets

network 192.168.20.0 0.0.0.3 area 0

network 2.2.2.2 0.0.0.0 area 0

network 192.168.10.0 0.0.0.255 area 0

!

router bgp 65010

bgp log-neighbor-changes

no synchronization

neighbor 192.168.20.2 remote-as 65000

network 192.168.20.0 mask 255.255.255.252

network 2.2.2.2 mask 255.255.255.255

redistribute ospf 1

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end

R2-H2#