R3-BRANCH>en

R3-BRANCH#sh run

Building configuration...

Current configuration : 1239 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R3-BRANCH

!

!

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX1524I8K6-

!

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback0

ip address 3.3.3.3 255.255.255.255

!

interface GigabitEthernet0/0

description LINK TO ISP

ip address 192.168.30.1 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/1

description LINK TO SW-BR

ip address 10.20.10.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

log-adjacency-changes

redistribute bgp 65020 subnets

network 10.20.10.0 0.0.0.255 area 1

network 192.168.30.0 0.0.0.3 area 1

network 3.3.3.3 0.0.0.0 area 1

!

router bgp 65020

bgp log-neighbor-changes

no synchronization

neighbor 192.168.30.2 remote-as 65000

network 192.168.30.0 mask 255.255.255.252

network 3.3.3.3 mask 255.255.255.255

redistribute ospf 1

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end

R3-BRANCH#